SMOBA ANOMALY LOGBOOK

Date                     Status                    Causes

05/26/98~06/11/98        missing         SBUV/2 Grating Drive Error

08/19/99                 missing         bad data dropout

04/01/01 --->          SBUV/2 switched   NOAA 14 crossing time too late,

to NOAA 16       no data in SH mid-high latitudes

05/03/01 --->          RTOVS swicthed    using same satellite (NOAA 16)

to ATOVS(N16)    for SBUV/2 and TOVS ozone

07/03/01 --->        Tuesday adjustment  missing SBUV/2 data (20E-100E)

every Tuesday

09/25/01 --->        Impact on SBUV/2    Update inter-range ratio (irr23)

11/09/01 --->            missing         bad data dropout

04/29/02 --->        Tuesday adjustment  neagtive values caused by linear

extrapolation of previous data

05/24/02 --->            missing         last orbit failure

11/22~11/30/02       No TOVS ozone used  NOAA-16 ATOVS problems

12/01/02 --->        Swithed to NOAA-15  same as above

TOVS ozone

01/01/03 --->            missing         bad data dropout

01/06/04 --->        Set O3 flag to 0-2  Previous setting is 0-4. New setting

excludes bad O3 at high zenith angle

11/11/04 --->        TOVS missing in NH  Program modified to handle the case

07/07~12/09/04       BUFR file delivered Analysis used pverious day's SBUV/2

to NCEP late        and current day's TOVS. Corrected.

01/07~01/12/05       No TOVS used        NOAA-16 maintanence

01/13~01/xx/05       NOAA-15 TOVS used   NOAA-16 maintanence

04/01/05 -->         TOAST TOVS used     Similar to NOAA-15 TOVS (NOAA TOVS

total O3 not available from previous

analysis form)

05/13/05             NOAA-17 data        NOAA-16 data missing

09/18/05 -->         Switch to NOAA-17   NOAA-17 better

11/18/05 -->         TOVS not used if    Too low ozone in south pole

SBUV/2 reach 80S/N